  - Grouping of WIM Sites Based on Axle Load Spectra

Based on the load spectra characteristics of steering axles and tridem axles, the grouping based on the tandem axle load spectra was refined into eight subgroups (Level 3 Groups), as shown in Table 1. If only the tandem axle load spectrum is to be considered for grouping, the WIM sites can be divided into Level 1 or Level 2 groups.

The decision tree that helps to determine which group a highway section should fall into is shown in Figure 1. The averaged traffic inputs from all WIM sites in each group were used as the traffic characteristics of the regions covered by the group. For CalME, the number of axles per truck and the hourly axle load spectra are averaged for each group. Other traffic inputs, including the number of axles per lane per year and growth rate, should be developed for each highway section from the Caltrans annual AADTT report (http://www.dot.ca.gov/hq/traffops/saferesr/trafdata/).

| Level 1 Groups | Level 2 Groups | Level 3 Groups | WIM Site |
| --- | --- | --- | --- |
| 1 | 1 | 1a | 011, 020, 040, 097, 057/058, 077/078, 079/080, 006, 022, 023, 026, 035, 036, 044, 045, 046, 065, 067, 068, 074, 081, 094, 003/004, 008/009, 012/013, 015/016, 017/018, 037/038, 041/042, 047/048, 051/052, 055/056, 059/060, 061/062, 082/083, 084/085, 095/096, 102, 103/104, 106, 848, 854, 856 |
|  |  | 1b | 014, 024, 031/032, 033/034, 039, 049, 063, 064, 076, 087/088, 089/090, 091/092, 093, 098, 099, 100/101, 107, 111/112 |
| 2 | 2a | 2aa | 001, 007, 027, 029, 050, 073, 105 |
|  |  | 2ab | 010, 043, 072, 075,113, 804, 828 |
|  | 2b | 2ba | 108, 812, 846 |
|  |  | 2bb | 005, 021, 066, 069/070, 110, 814 |
| 3 | 3 | 3a | 002, 028, 030 |
|  |  | 3b | 025, 071 |

Table 1 WIM Site Grouping Based on All Axle Load Spectra

.

Figure 1 Flowchart for grouping highways based on axle load spectra

The number of axles per lane per year can be calculated based on the following equation:

(6)

where,

= of axles per lane per year,

= base year Annual Average Daily Truck Traffic (2-way),

= number of axles per truck,

= directional distribution factor of truck traffic (Table 2, default value = 0.52),

= lane distribution factor of truck traffic, determined from Table 3.

The growth rate of truck traffic can be estimated from the historical AADTT data compiled in the Caltrans annual report, or obtained from the statewide averages developed from WIM data, as shown in Table 4.

| Group | Directional Distribution Factor |
| --- | --- |
| 1 | 0.519 |
| 1a | 0.515 |
| 1b | 0.527 |
| 2 | 0.516 |
| 2a | 0.514 |
| 2aa | 0.515 |
| 2ab | 0.509 |
| 2b | 0.520 |
| 2ba | 0.509 |
| 2bb | 0.522 |
| 3 | 0.521 |
| 3a | 0.512 |
| 3b | 0.533 |

Table 2 Directional Distribution Factor

| Group | No. of Lanes | Average LDF | Min. LDF | Max. LDF |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1.000 | 1.000 | 1.000 |
| 1 | 2 | 0.869 | 0.532 | 0.970 |
| 1 | 3 | 0.671 | 0.513 | 0.874 |
| 1 | 4 | 0.589 | 0.470 | 0.839 |
| 1 | 5 | 0.553 | 0.521 | 0.584 |
| 1 | 6 | 0.496 | 0.496 | 0.496 |
| 2 | 1 | 1.000 | 1.000 | 1.000 |
| 2 | 2 | 0.909 | 0.863 | 0.943 |
| 2 | 3 | 0.671 | 0.536 | 0.757 |
| 2 | 4 | 0.522 | 0.522 | 0.522 |
| 3 | 1 | 1.000 | 1.000 | 1.000 |
| 3 | 2 | 0.931 | 0.917 | 0.944 |

Table 3 Lane Distribution Factor for lanes with heaviest truck traffic

| Truck Class | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | All |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Linear Growth Rate (Relative to 2000 AADTT),% | 4.92 | 6.53 | 2.92 | 2.93 | 0.23 | 3.99 | 4.38 | 0 | 0.35 | 0.41 | 3.97 |
| Compound Growth Rate,% | 5.95 | 8.00 | 3.19 | 2.75 | 0.45 | 4.48 | 4.55 | 0 | 0.55 | 1.44 | 4.41 |

Table 4 Average Truck Traffic Growth Factors (percent growth in truck volume per year, linear growth)

If the AADTT is unavailable in the Caltrans annual report, it is suggested that an onsite count of truck traffic be conducted for two separate hours. Trucks with 2, 3, 4, 5 and more axles should be counted separately. AADTT can then be estimated from the following formula:

(7)

where,

= number of trucks counted in one hour (Hour i, i =1, 2) in one direction,

= hourly distribution factor of truck traffic volume for Hour i (i =1, 2),

= Monthly distribution factor of truck traffic volume for Month m (m =1, 2).

The hourly and monthly distribution factors for each group (Level 1) can be found in Table 5 and Table 6, respectively.

| Hour | Group 1 | Group 2a | Group 2b | Group 3 |
| --- | --- | --- | --- | --- |
| 1 | 1.430 | 2.287 | 2.670 | 2.496 |
| 2 | 1.341 | 2.108 | 2.480 | 2.206 |
| 3 | 1.389 | 2.094 | 2.428 | 2.055 |
| 4 | 1.695 | 2.348 | 2.478 | 2.026 |
| 5 | 2.481 | 2.794 | 2.608 | 2.132 |
| 6 | 3.733 | 3.499 | 2.890 | 2.466 |
| 7 | 5.003 | 4.135 | 3.455 | 3.039 |
| 8 | 5.450 | 4.500 | 3.959 | 3.801 |
| 9 | 6.228 | 4.936 | 4.419 | 4.494 |
| 10 | 6.866 | 5.458 | 4.970 | 5.057 |
| 11 | 7.099 | 5.771 | 5.347 | 5.450 |
| 12 | 7.173 | 5.905 | 5.534 | 5.626 |
| 13 | 7.014 | 5.947 | 5.673 | 5.694 |
| 14 | 6.920 | 5.916 | 5.774 | 5.709 |
| 15 | 6.660 | 5.905 | 5.788 | 5.780 |
| 16 | 6.070 | 5.771 | 5.661 | 5.791 |
| 17 | 5.096 | 5.352 | 5.416 | 5.649 |
| 18 | 4.124 | 4.801 | 5.089 | 5.445 |
| 19 | 3.415 | 4.367 | 4.616 | 5.182 |
| 20 | 2.805 | 3.868 | 4.294 | 4.827 |
| 21 | 2.386 | 3.486 | 4.131 | 4.452 |
| 22 | 2.104 | 3.198 | 3.834 | 4.049 |
| 23 | 1.875 | 2.927 | 3.453 | 3.565 |
| 24 | 1.645 | 2.626 | 3.032 | 3.006 |

Table 5 Hourly Distribution Factor (percent of truck traffic in each hour)

|  | Group 1 | Group 2 | Group 3 | Mexico Border |
| --- | --- | --- | --- | --- |
| 1 | 0.893 | 0.866 | 0.854 | 1.205 |
| 2 | 0.912 | 0.879 | 0.892 | 1.330 |
| 3 | 0.949 | 0.927 | 0.936 | 1.270 |
| 4 | 0.989 | 1.003 | 0.958 | 1.045 |
| 5 | 1.020 | 1.018 | 1.026 | 1.135 |
| 6 | 1.041 | 1.044 | 1.040 | 1.020 |
| 7 | 1.055 | 1.066 | 1.078 | 0.770 |
| 8 | 1.081 | 1.109 | 1.082 | 0.680 |
| 9 | 1.061 | 1.096 | 1.076 | 0.710 |
| 10 | 1.034 | 1.061 | 1.098 | 0.780 |
| 11 | 0.990 | 0.991 | 1.006 | 0.900 |
| 12 | 0.972 | 0.948 | 0.956 | 1.155 |

Table 6 Monthly Distribution Factor (truck traffic relative to average monthly traffic)